IMX541 MIPI camera HW SOW

- The camera should have 2 PCBS a FPGA PCB and sensor PCB

- All the new components should be approved by KAYA

- Sensor PCB specification

  - The sensor should be IMX541

  - The sensor should operate on SLVS-EC mode

  - All the sensor voltages should be generated on sensor PCB

  - The connection between sensor PCB and FPGA PCB should be as in IRON FPGA board

  - Board size should be the same as IRON FPGA board

- FPGA PCB specification

  - Based on Polarfire MPF100T

  - An IRON FPGA board should be taken as a starting point

  - A Coaxpress interface should be replaced with MIPI CSI interface

  - The MIPI should use FPGA transceivers

  - MIPI pinout should be as below

  - MIPI connector should be I-PEX  20682-030E-02

  - The voltages towards the sensor board should be kept the same , If needed a change should be coordinated with KAYA

  - Input voltage is 5V +-20%

  - Board size should not changed unless coordinated with KAYA

Figure 1: MIPI Pinout

MCLK and PWDN are 1.8V levels , I2C and GPIO are 3.3V levels